99-channel multi-function wireless caller server

Operation Instruction

I Summary

99-channel wireless caller server adopts advanced micro processor technology. It is not only the caller server, but also a 99-area wireless alerter. The products are useful for public remote control lock, as well as resident area. The administrator can add or delete other user accounts, and add or remove the mini remote controller. The encode uses code studying technology which has millions group of different code. So it has a high security. The products are widely used in security guard of schools, factories, offices, building and resident area; and caller service of restaurants, cafés, hotels, pubs and hospitals.

II Technical Data

Working voltage: DC12V

Working frequency: 433.92MHz

Transmit power: > 100mW

Receive sensitivity: > -112dBm

III Code Studying Operation

1. Clear all the records: when you first use it, please clear all the programmed callers and calling records. ① Press and hold on the key “STUDY” for 5 seconds, the digitron displays “000” and flicker one time, it clears all programmed callers. ② Press key and hold on “CLEAR” in control panel for 5 seconds, the digitron displays “---” and flicker one time, it clears all the calling records. ③ Press the key “CLEAR”, the digitron displays “---” to clear screen. ④ Clear some one of the callers: enter the status of studying, press key “UP’ to find out the one you will delete, and press key “CLEAR”, then the digitron displays “F”, it delete the caller, press “STUDY” to quit.

2. Program caller: press the key “STUDY” to enter code studying status. Press key “UP” the digitron displays “01-”, then press any button on the caller, the digitron will display “01F”, it means the caller has been programmed in the host. About 0.5 seconds later, the digitron will jump to “02-” automatically which means that it will make another caller code. The maximum is 99. If you want keep some one number empty, press key “UP” jump it to continue learning others caller. The encode method of 4-key caller is same to 2-key caller. But you should select right position of internal toggle switch. In some special occasion, you just can use 1-key caller.

3. Program management remote controller: press key “STUDY” on host to enter studying status, it displays “00-”, press any key on management remote controller, the digitron displays “00F” for 1 seconds and turns to “00-” automatically, the management remote controller is programmed successfully. You can continue to study others management remote controller, the maximum is 5, if you learnt the 6th controller, the 1st one will be cleared.

When programmed the management remote controller, you can use this controller to browse/clear screen/clear zero the host.

The management remote controller must be used in public wireless door control system.

4. Program wireless alarm remote controller: please refer to item 3, but select the right position of toggle switch. For the method of wireless door magnetic, wireless infrared detecting, please refer to item 2 (can program 01-99 detectors). But the wireless alerter should be learnt in 00 position of placing or undo security system.

5. The program method of opening door remote controller: please refer to item 2. Please notice the toggle switch’s position and learn on channel 1-99.

6. Query the calling records: press key “UP” to browse the latest 9 records, if there is no records or the records have been cleared, the digitron will display “-” while page up.

IV Changing Number

Sometimes you want change some one of the callers or unlock remote controller, you can use the method of covering to clear the old code and record the one. In the status of studying, press key “UP” to find out the number you want change, press any key of the new caller or unlock remote controller to study. The digitron will display “F”, and then press the key “STUDY” to quit. The settings is finished.

Note: press and hold the key “UP” to browse records quickly.

V Music Selection

1. Press and hold on key “Music selection” for 5 seconds, the digitron displays “(01~16)-”, press page up key to select one song. There are 16 songs in total. And then press and hold on key “Music selection” for 5 seconds, the digitron displays “(01~16)F” which shows the settings is finished.

2. Testing – Press the key “Music selection”, it plays the songs selected.

VI Notice

1. Before making music selection, please power off once.

2. When you use the toggle switch, please power off the host.

3. Key “STUDY”, “UP” have same function with the keys in remote controller.

4. While using as an alerter, the manager function disappears.

5. Screen clearing just clears the screen, the records are still in, press page up to browse; Zero clearing clears the screen and records.

6. Alarm time is 2 minutes as an alerter.

7. Please pull the antenna of the remote controller and host, the effect of transmitting and receiving will be better.

Memo: function of internal toggle switch

① double key caller transmitter ②Manage remote controller ③Wireless place or undo security system/open door remote controller.

Toggle switch is in upper left corner of the display screen.

# ON

# 1  2  3

# ON

# 1  2  3

# ON

# 1  2  3

# ON

# 1  2  3

①single/double key caller function ② 4-key caller function ③99-channel wireless alerter function ④ door remote control management function

Instruction: the meanings of the buttons on 4-key desktop caller are defined by user. The buttons 1~3 display “desk no. +A/B/C”, the last button is cancel function, as well as the last button on 2-key desktop caller, press this button, it will display “00(0-9)” which shows cancel the service from this desk.

Above diagram show how to use it as caller and alerter. When it is used to unlook, just key B works.